Comparison of Attached DoA and Metasurface Contributions

Scenario, frequency, waveform/standard, reported accuracy, and comparison reference

Scope. Living table for the six attached papers. The last column states both the result and what it is compared against. Last updated: 22 July 2026.

| Reference | Scenario and validation | Frequency | Standard / waveform | Reported positioning or angular performance, including comparison reference |
| --- | --- | --- | --- | --- |
| Şenyuva, 2026 Paper: Covariance-Guided DFT Beam Selection for Beamspace ESPRIT | Validation: Numerical Monte Carlo only. Scenario: Generic far-field narrowband mmWave model; 32-element ULA and three uncorrelated paths. Not specifically indoor or outdoor. | Carrier: Not specified. Model: Wavelength-normalized. | Standard: None. Waveform: Narrowband observation, interpretable as one OFDM subcarrier. | Reported: About 1-2 dB from the stochastic CRB for ASNR at or above 4 dB; below 10% failure around 0-1 dB ASNR. No degree-valued resolution or position error. Compared against: Sectorization-based beamspace ESPRIT with the same codebook, estimator, and beam budget; coarse-only stages; oracle beams; and the CRB. Sectorization needs roughly 4-6 dB more ASNR for a similar near-CRB regime. |
| Zheng et al., 2026 Paper: 1-Bit Amplitude-Adjustable Metasurface With Spatio-Temporal Modulation | Validation: Numerical and indoor laboratory. Scenario: Far-field horn test, fabricated 6 x 6 transmissive surface, and one receiver. | Carrier: 5 GHz. Modulation: 2.0-2.5 MHz. | Standard: None. Waveform: Continuous wave, amplitude time modulation, harmonics, and MUSIC. | Reported: Experimental 1-D DoA error below 0.61 degrees over 0-60 degrees from 50 repeated measurements. Compared against: Prior metasurface DoA systems in Table II, mainly phase-modulated/reflective designs and an active-amplifier transmissive design. The comparison is qualitative across published systems, not a controlled same-setup RMSE test. |
| Li et al., 2024 Paper: Direct AOA Estimation With Single-Frequency Phaseless Measurements | Validation: Numerical and indoor laboratory. Scenario: Far-field horn, 2-bit transmissive surface, fixed probe, and intensity-only measurement. | Experiment: 5.8 GHz. Simulation: Wavelength-normalized. | Standard: None. Waveform: Single-tone continuous wave and analytical phaseless spatial matched filtering. | Reported: Simulation 3-dB width: 5.54 degrees versus 5.08 degrees from aperture theory. Experiment: about 10.9 degrees versus 8.75 degrees theoretical; 200 measurements take about 0.31 s. Compared against: OMP-based compressive sensing and least squares under fixed measurement budgets and increasing target count. CS is sharper when sparsity holds, but can produce false/displaced targets and may need up to 850 measurements for four targets. Theory and simulation are also used as references. |
| Guo et al., 2026 Paper: DOA Estimation With Compressive Measurements and Mixed-ADCs | Validation: Numerical only. Scenario: Generic far-field narrowband array; 12 elements compressed to eight RF channels. Not specifically indoor or outdoor. | Carrier: Not specified. Model: Normalized DoA. | Standard: None. Waveform: Narrowband sources, spatial compression, mixed-resolution ADCs, and MUSIC. | Reported: Five and six sources resolved at 20 dB SNR with 100 snapshots. Mixed-ADC RMSE lies between all-high- and all-low-resolution cases; maximum reported degrees of freedom remains 25. Compared against: Traditional compressed arrays with all high-resolution or all low-resolution ADCs, using MUSIC for all schemes; simulated RMSE versus the derived CRB; and the earlier compressed sparse-array CRB. |
| Bai et al., 2023 Paper: 1-Bit Programmable Metasurface-Based 2-D Direction Finding | Validation: Numerical and indoor laboratory. Scenario: Far-field horn and turntable. Experiment validates elevation only; full 2-D operation is numerical. | Simulation: 2.4 GHz. Experiment: 29 GHz. | Standard: None. Waveform: Single-tone continuous wave, time modulation, harmonic ratios, and FFT. | Reported: Experimental direction-finding MSE about 0.21 degrees for 14 angles from 0 to 6.5 degrees. Numerical azimuth/elevation MSE approaches zero at high SNR. Compared against: Preset turntable angles and the analytical harmonic-ratio mapping. Related MUSIC and OMP methods are discussed, but no matched head-to-head estimator baseline is presented. |
| Fang et al., 2024 Paper: Multifunctional Space-Time-Modulated Metasurface for DoA and RCS | Validation: Analytical, Monte Carlo, CST full-wave, and indoor X-band experiment. Scenario: Controlled horn/plane-wave measurements with a fabricated surface. | Carrier: 9 GHz. Modulation: 1 kHz experimentally. | Standard: None. Waveform: Monochromatic continuous wave, binary modulation, harmonics, and MUSIC. | Reported: Experimental DoA error below 2 degrees over -50 to +50 degrees. Numerical MSE below 0.3 degrees over about -80 to +80 degrees at 10 dB; nonideal-state error below about 0.5 degrees. Compared against: Known incident angles; analytical results versus full-wave simulation and experiment; and ideal states versus finite-isolation nonidealities. Prior dual-functionality is discussed, but no same-scenario accuracy baseline against another DoA estimator is given. |

Reading notes

• None of the six papers reports Cartesian positioning error in meters or centimeters; outcomes are angular error, RMSE/MSE, 3-dB beamwidth, failure rate, or gap to a bound.

• Ground-truth or theory validation is not the same as a head-to-head estimator comparison; the last column distinguishes these cases.

• This is the master document for subsequent additions and corrections.